# 3D Printing Summary

| Metrics |  |
| --- | --- |
| Total Print Time (min) | 224 - 241 |
| Total Number of Components | 1 - 5 |
| Typical Total Mass (g) | 36 - 39 |
| Typical Number of Print Setups | 1 - 5 |

# 3D Printing Settings

| Print File Name | Qty | Total Print Time (hr:min) | Mass (g) | Infill (%) | Support | Layer Height/ Nozzle Diameter(mm) | Notes (orientation, special settings, etc.) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bottle_Opener_30.stl | 1 | 3:47 | 37 | 50 | No Support | 0.2/0.4 | Adjust number of perimeters to 3-4. |
| Bottle_Opener_35.stl | 1 | 3:44 | 36 | 50 | No Support | 0.2/0.4 | Adjust number of perimeters to 3-4. |
| Bottle_Opener_40.stl | 1 | 3:46 | 36 | 50 | No Support | 0.2/0.4 | Adjust number of perimeters to 3-4. |
| Bottle_Opener_45.stl | 1 | 3:51 | 38 | 50 | No Support | 0.2/0.4 | Adjust number of perimeters to 3-4. |
| Bottle_Opener_50.stl | 1 | 4:01 | 39 | 50 | No Support | 0.2/0.4 | Adjust number of perimeters to 3-4. |

# Post-Processing

- Remove any burrs or sharp edges that may occur if applicable.

# Customization Options

- There are parameters given in the CAD file (.f3d) to change the cap size and handle size if desired.

- Can be printed in any colour.

# Examples of Quality Prints:

Example of Bottle Opener 40

Example of Bottle Opener 40

Example of Bottle Opener 40